Creator: Elena Cantero; Oscar Pires; Alvaro González

Reviewed on: 29/10/2025

License: Only for purposes of the AIRE project.

DISDROMETER

Description: 1-minute raw data from disdrometer (OTT Parsivel2) located in Alaiz CENER’s experimental wind farm. netCDF4 file format, which contains all recorded data. Two precipitation characterization campaigns were performed. AIRE project.

Time period:

First campaign: From 16 February to 16 November 2023.

Second campaign: From 18 July 2024 to 09 May 2025

Files:

| File_name | Description | Size | # Files | Dates |
| --- | --- | --- | --- | --- |
| CENER_Disdrometer_AIRE_Raw.nc | 1-minute raw data registered with a disdrometer | 7 MB | 1 | February to November 2023 |
| CENER_Disdrometer_AIRE_Raw20242025.nc | 1-minute raw data registered with a disdrometer | 11 MB | 1 | July 2024 to May 2025 |

For each field in netCDF4 file:

| Variable name | Dimensions | Description | Unit |
| --- | --- | --- | --- |
| diameter_bin_center | diameter_bin_center | Drop size distribution class bins | mm |
| velocity_bin_center | velocity_bin_center | Fall velocity distribution class bins | m/s |
| time | time | Time in minutes measured since 2023-02-16 10:45:00 | minutes |
| Rain_intensity | time | Rain intensity | mm/h |
| Rain_amount_accumulated | time | Rain amount accumulated | mm |
| Weathe_code_acc_to_SYNOP_Table_4680 | time | Precipitation type according to SYNOP table 4680 | --- |
| Weathe_code_acc_to_SYNOP_Table_4677 | time | Precipitation type according to SYNOP table 4677 | --- |
| Radar_reflectivity | time | Radar reflectivity | dBZ |
| MOR_visibility_in_precipitation | time | MOR visibility in precipitation | m |
| Signal_amplitude_of_the_laser_strip | time | Signal amplitude of the laser strip | --- |
| Number_of_particles_detected_and_validated | time | Number of particles detected and validated | --- |
| Temperature_in_the_sensor_housing | time | Temperature in the sensor housing | °C |
| Heating_current | time | Heating current | A |
| Power_supply_voltage | time | Power supply voltage | V |
| Sensor_status | time | Sensor status | --- |
| Station_number | time | Station number | --- |
| Temperature_in_the_right_sensor_head | time | Temperature in the right sensor head | °C |
| Temperature_in_the_left_sensor_head | time | Temperature in the left sensor head | °C |
| Kinetic_energy | time | Kinetic energy | J/(m²h) |
| Snow_depth_intensity | time | Snow depth intensity | mm/h |
| Field_N_d | time, diameter_bin_center | Drop density of the drop size distribution | log₁₀(1/(m² mm)) |
| Field_v_d | time, velocity_bin_center | Average rate of fall per class | m/s |