AIRE MRR ReadMe -Risø

Description: Raw and processed data from METEK MRR-PRO located in Risø.  The instrument was configured to read precipitation information using doppler reflectivity information. The data is stored in netCDF4 file format. Each file contains an hour of recorded data at a frequency of 5 seconds.

Instrument information:

| Instrument | MRR |
| --- | --- |
| Working | Shoots doppler signals vertically and records backscatter reflections from rain. |
| Resolution of data | 5 sec |
| Heights of measurement | Different heights from 50m to 3000m |
| Location of instrument | Risø met mast |
| Available years of measurements | Available from 2020 |
| Output file | netCDF |

Time period: 1st January 2023 to 31st December 2024

Files:

Data files are stored in daily folders, which are localized in monthly folders and yearly folders. Each day folder contains raw and processed files.

The folder naming convention is:

- For daily folders ‘YYYYMMDD’

- For monthly folders ‘YYYYMM’

- For yearly folders ‘YYYY’

Where YYYY, MM, DD, hh, mm, and ss are year, month, day of month, hour, minute and second, respectively.

| File_name | Description | Size |
| --- | --- | --- |
| YYYYMMDD_hhmmss.nc | MRR raw data | 0.5 to 70 MB |
| YYYYMMDD_hhmmss-processed_M_C.nc | MRR RaProM-Pro post-processed data | 0.5 to 70 MB |

Post-processing information:

The post-processing of the raw data was performed in 3 steps:

Processing step 1: Processing with M equals one results in an extreme overestimation of the total precipitation. Multiple trials were conducted on the raw data by computing the multiplicative bias from different approaches. But all the results showed large deviation around 200% larger than the raw data.

Processing step 2: To tackle this issue, the overestimation from the processing step is removed by re-processing the data with a new multiplicative bias. This new bias is computed by comparing the results of total precipitation from the first processing step to the rain gauge data. The following steps are followed to compute the new M:

- Validate the processed files. Some files show a mismatch in time (The size of time stored in the .nc file and size of the precipitation data do not match) and hence all data in that 1hr time frame will be excluded.

- The time series of precipitation from the MRR at a height of 150m and the rain gauge at the ground for one day is extracted. (Note: The rain gauge data from the time frame where MRR files do not exist are ignored)

- The resolution of the extracted MRR data is then changed from 5 sec to 10 min for direct comparison with rain gauge data.

- After resampling, any readings in the MRR less than 0.2mm of total precipitation are assumed to be zero to prevent that these small reading affect the computation of M.

- The final M value is computed for each day. This is assumed to be the optimal resolution at which the lag from the rainfall due to measurements at different heights cannot be observed.

Processing step 3: All the files in one day are processed using the calculated M value. The reason for not using a higher resolution is mentioned above. If the rain gauge data from the day showed no rainfall, all the files from the day were processed with M=1 to prevent division by zero during the calculation. The same measure is used if the total precipitation of the MRR for the day is 0 mm.

For each field in netCDF4 raw files:

| Variable name | Dimensions | Description | Unit |
| --- | --- | --- | --- |
| range | range | Height in meters | m |
| time | time | Time in unix format (seconds since1970-01-01T00:00:00Z) | s |
| index_spectra | time, range | 'index variable spectra' | minutes |
| spectrum_raw | time, n_spectra, spectrum_n_samples | log_attenuated_power | mm/h |
| WIDTH | time, range | Doppler spectrum width | mm |
| PIA | time, range | Path integrated rain attenuation | dB |
| SNR | time, range | Signal to noise ratio | dB |
| Za | time, range | Log attenuated reflectivity | dBZ |
| Z | time, range | Log reflectivity | dBZ |
| Ze | time, range | Equivalent reflectivity factor | dBZ |
| Zea | time, range | Attenuated equivalent reflectivity factor | dBZ |
| D | n_spectra,spectrum_n_samples | Drop size | mm |
| N | time, n_spectra, spectrum_n_samples | Drop size distribution | log₁₀(1/(m3 mm)) |
| VEL | time, range | Radial velocity of scatterers towards instrument | m/s |
| RR | time, range | Rainfall rate | mm/h |
| LWC | time, range | Mass concentration of liquid water in air | g/m3 |
| ML | time, range | Melting layer | m |

For each field in netCDF4 processed files:

| Variable name | Dimensions | Description | Unit |
| --- | --- | --- | --- |
| Height | Height | Height in meters | m |
| time | time | Time in unix format (seconds since1970-01-01T00:00:00Z) | s |
| time_utc | time_utc | time UTC ('seconds since 2023-02-23 02:00:00') | S |
| spectral width | time_utc, Height | Spectral of the spectral reflectivity width with dealiasing | m/s |
| Skewness | time_utc, Height | Skewness of the spectral reflectivity with dealiasing | - |
| Kurtosis | time_utc, Height | Kurtosis of the spectral reflectivity with dealiasing | - |
| DBPIA | time_utc, Height | Path Integrated Attenuation, expressed as 10*log(PIA) without hydrometeor type consideration | dB |
| SNR | time_utc, Height | Signal noise relation from signal without dealiasing | dB |
| Noise | time_utc, Height | Noise | m/s |
| Za | time_utc, Height | Attenuated Reflectivity considering only liquid drops | dBZ |
| Z | time_utc, Height | Reflectivity considering only liquid drops | dBZ |
| Z_all | time_utc, Height | Reflectivity supposing that all hydrometeors are in liquid phase | dBZ |
| Ze | time_utc, Height | Attenuated Equivalent Reflectivity | dBZ |
| Zea | time_utc, Height | Equivalent Reflectivity attenuated | dBZ |
| DropSize | DropSize | Size of the water drops | mm |
| dsd_3D | time_utc, Height, DropSize | 'Drop Size Distribution in function of time, height and diameter' | log₁₀(1/(m3 mm)) |
| dsd_3D_full | time_utc, Height, DropSize | Drop Size Distribution in function of time, height and diameter, independent of type' | log₁₀(1/(m3 mm)) |
| N(D) | time_utc, Height | Drop Size Distribution | log₁₀(1/(m3 mm)) |
| N(D)_all | time_utc, Height | Drop Size Distribution supposing that all hydrometeors are in liquid phase | log₁₀(1/(m3 mm)) |
| Nw | time_utc, Height | Intercept parameter of the gamma distribution normalized to the liquid water content | log₁₀(1/(m3 mm)) |
| Dm | time_utc, Height | Mean mass-weighted raindrop diameter | mm |
| Nw_all | time_utc, Height | Intercept parameter of the gamma distribution normalized to the liquid water content supposing that all hydrometeors are liquid phase | log₁₀(1/(m3 mm)) |
| Dm_all | time_utc, Height | Mean mass-weighted raindrop diameter content supposing that all hydrometeors are liquid phase | mm |
| Dm_ax | Dm_ax | Mean diameter axes of precipitation regime | mm |
| Nw_ax | Nw_ax | Intercept parameter of precipitation regime | log₁₀(1/(m3 mm)) |
| DM_ax | DM_ax | Mean diameter axes of precipitation regime supposing all hydrometeors are in liquid phase | (mm) |
| NW_ax | NW_ax | Intercept parameter axes of precipitation regime supposing all hydrometeors are in liquid phase | log₁₀(1/(m3 mm)) |
| W | time_utc, Height | 'Fall speed with aliasing correction' | m/s |
| Speed | Speed | Speed without aliasing | m/s |
| RR_all | time_utc, Height | Rain Rate supposing that all hydrometeors are in liquid phase | mm/h |
| RR | time_utc, Height | Rain Rate | mm/h |
| SR | time_utc, Height | Snow Rate | mm/h |
| LWC | time_utc, Height | Liquid water content | g/m3 |
| LWC_all | time_utc, Height | Liquid water content supposing that all hydrometeors are in liquid phase | g/m3 |
| BB_bottom | time_utc, BB_Height | Height of BB bottom in meters a.g.l. | m |
| BB_top | time_utc, BB_Height | Height of BB top in meters a.g.l. | m |
| BB_peak | time_utc, BB_Height | Height of BB peak in meters a.g.l. | m |
| Type | time_utc, Height | Type of hydrometeor: unknown (20), rain (10), drizzle (5), mixed (0), snow (-10), graupel (-15) and hail (-20) | - |
| TyPrecipi | Dm_ax Nw_ax | Precipitation regime values: convective (5), transition (0) and stratiform (-5) | - |
| TyPrecipi_all | DM_ax NW_ax | Precipitation regime values: convective (5), transition (0) and stratiform (-5) supposing all hydrometeors are in liquid phase | - |
| PIA_Height | PIA_Height | Heights in meters a.s.l. | m |
| BB_Height | BB_Height | Heights in meters a.s.l. | m |