Description: Data in this directory are measured on the Risø site (55 41 39.1 N,12 05 17.7 E in WGS 84). The site is located on the coast of the Roskilde fjord, on the Danish island of Sjælland.

Access: Open

Creator: Ebba Dellwik, ebde@dtu.dk

License: CC BY 4.0

Description: 10 minute mean value data from a WindCube V2 (Vaisala Oy) lidar placed 122m balcony of the Risø tower at 122m height. The resulting data set was split into two parts, reflecting an adjustment of measurement height at 2023-04-04. There two files include the same parameters. Data have been adjusted in three ways in relation to the raw data stream:

(1) The wind direction was adjusted to reflect that the WindCube was oriented towards 45 degrees.

(2) Ten minutes were added to the time WC stamp, such that the WindCube and mast data are synchronized. This was necessary since the time stamp by the WindCube reflects the start of the ten-minute measurement period, whereas the time stamp of the met mast reflects the end of the measurement period. The stamp reflects UTC+1, end of 10 minute measurement period.

(3) 122m has been added to the original measurement heights, to account for the height of the balcony at the top of the Risø tower.

You can read more about the instrument in this document WindCube Data Filtering Guidelines (vaisala.com).

Time period: 2023-02-02 – 2023-12-12

| File_name | Description | Size | # Files | Dates |
| --- | --- | --- | --- | --- |
| WC_Risoe_10minutes_heights1 | First part of dataset | 31.2 MB | 1 | 2023-02-02 -- 2023-04-04 |
| WC_Risoe_10minutes_heights2 | Second part of dataset | 132.9MB | 1 | 2023-04-04 -- 2023-12-12 |

The files contain the following measurements

| Field_name | Description | Variable | Unit |
| --- | --- | --- | --- |
| Timestamp | Date-Time in UTC+1: Year Month Day Hour Minute | Time |  |
| Height above local ground level | See point (3) under Description | Height | m |
| Availability | Availability within 10 minute period | Availability | % |
| Mean wind speed | 10 minutes mean value of wind speed | Wind speed | m/s |
| Std wind speed | 10 minutes standard deviation of wind speed | Wind speed | m/s |
| Min wind speed | 10 minutes minimum value of wind speed | Wind speed | m/s |
| Max wind speed | 10 minutes maximum value of wind speed | Wind speed | m/s |
| Mean wind direction | 10 minutes mean value of wind direction | Wind direction | º |
| Std wind direction | 10 minutes standard deviation of wind direction | Wind direction | º |
| Min wind direction | 10 minutes minimum value of wind direction | Wind direction | º |
| Max Wind direction | 10 minutes maximum value of wind direction | Wind direction | º |
| CNR mean | 10 minute mean value of signal strength | Carrier to Noise Ratio | dB |
| CNR std | 10 minute standard deviation of signal strength | Carrier to Noise Ratio | dB |
| CNR min | 10 minute minimum value of of signal strength | Carrier to Noise Ratio | dB |
| CNR max | 10 minute maximum value of of signal strength | Carrier to Noise Ratio | dB |